THERMODYNAMICS I REVIEW SHEET

Energy Balance – Closed Systems

E2-E1=Q-W

Energy Rate Balance – Closed

dEdt=Q-W

Energy Rate Balance – Steady State

0=QCV-WCV+imihi+Vi22+gzi-emehe+Ve22+gze

Energy Balance – Power Cycle

Wcycle=Qin-Qout

Thermal Efficiency – Power Cycle

η=WcycleQin

Energy Balance – Refrigeration/Heat Pump Cycle

Wcycle=Qout-Qin

Coefficient of Performance – Refrigeration

β=QinWcycle

Coefficient of Performance – Heat Pump

γ=QoutWcycle

Closed System Entropy Balance

S2-S1=12δQTb+σ

Steady State Control Volume Entropy Rate Balance

0=jQjTj+imisi-emese+σCV

Ideal Gas Relations – Constant Specific Heat ratio, k, s1=s2

T2T1=p2p1k-1k                     T2T1=v1v2k-1                   p2p1=v1v2k

Ideal Gas Relation for Entropy – Constant Specific Heat

sT2,p2-sT1,p1=cplnT2T1-Rlnp2p1

Isentropic Efficiency

Compressor & Pump

ηc=-WCVms-WCVm=h2s-h1h2-h1

Turbine

ηt=-WCVm-WCVms=h1-h2h1-h2s

Saturation Table – Property Relationship (use for v, u, h, & s)

u1=uf+x1ug-uf

Tds Relations

Tds=du-pdv

Tds=dh-vdp